ELECENG 3TP3

Lab 3: Aliasing in Signal Sampling

Instructor: Dr. Kiruba

Jasmine Dosanjh – dosanj5 – 400531879

Warisha Noushad – noushadw – 400519903

December 2nd, 2025

# Question 1: Aliasing in the Telephone System

## Part 1.

The MATLAB code below takes the analog (continuous-time) sinusoid voice input:

xt=sin⁡(2πft+ϕ)

and samples it at periodic intervals of the sampling period, creating the discrete-time sampled signal:

xn=xnTs=sin⁡(2πnf/fs+ϕ)

This script initially assigns a sinusoid xt=sin⁡(2πft) with frequency f=100 Hz. To create the sampled signal, a sampling rate fs=8000 Hz is defined, with the corresponding sampling period Ts=1fs=18000=0.000125 s. Samples are taken at periodic intervals of the sampling period Ts. This is implemented through the vector nplot, which goes from 0 to 10 ms in steps of Ts: nTs=0, 0.000125, 0.00025,..., 0.01. Lastly, the sampled sinusoid xnTs is plotted using the stem command and exported.

Listing 1: Part 1

1. %%Question 1

2. % Do a plot of a sampled sinusoid with frequency f = 100 Hz

3. f = 100

4. % Sampling frequency and interval

5. fs = 8000;

6. Ts = 1/fs;

7. % Set time duration of plot, i.e., 10 msec.

8. tfinalplot = 10e-3;

9. % Make the time vector for the plot

10. nplot=0:Ts:tfinalplot;

11. % Sample the sinusoid.

12. xnT = sin(2*pi*f*nplot);

13. % Make the plot

14. stem(nplot, xnT);

15. text(6.5e-3,0.9, 'Jasmine Dosanjh - 400531879', 'FontSize',8)

16. text(6.5e-3,0.8, 'Warisha Noushad - 400519903', 'FontSize',8)

17. xlabel('Time (s)');

18. ylabel('Amplitude');

19. title('Sampled Sinusoid at 100 Hz');

20. %Export the graph

21. exportgraphics(gcf, "Q1.jpg")

The output discrete-time sinusoid is shown below. It exists for a duration of 10 ms, with an amplitude of 1, and no phase shift.

Figure 1. Discrete-Time Sinusoid at frequency 100 Hz

## Part 2.

The MATLAB code in Listing 2 observes the effect on the discrete-time signal xn from Part 1 at varying frequencies. Lines 1-10 remain the same as Part 1, to create xn and generate the short-time vector nplot. A second, longer time vector, nsound, is created to generate a 2-second audio tone for each frequency.

The sinusoid is then sampled at four frequencies, f=100, 200, 400, 800 Hz. For each frequency, a discrete-time signal x[n] is created. These four tone segments are concatenated sequentially into a single 8-second vector sound_created. All four sinusoids are plotted using the subplot command and exported.

Listing 2: Part 2

1. %% Question 2

2. % Use sinusoid frequency f = 300 Hz

3. %f = 100

4. % Sampling frequency and interval

5. fs = 8000;

6. Ts = 1/fs;

7. % Set time duration of plot, i.e., 10 msec.

8. tfinalplot = 10e-3;

9. % Make the time vector for the plot

10. nplot=0:Ts:tfinalplot;

11. % Make the time vector for replayed sound spurt

12. % Play the spurt for 2 seconds

13. tfinal = 2;

14. nsound=0:Ts:tfinal;

15. %Frequency for sin

16. f1 = 100;

17. f2 = 200;

18. f3 = 400;

19. f4 = 800;

20. % Sample the sinusoid.

21. xnT1 = sin(2*pi*f1*nsound);

22. xnT2 = sin(2*pi*f2*nsound);

23. xnT3 = sin(2*pi*f3*nsound);

24. xnT4 = sin(2*pi*f4*nsound);

25. %Plot Making

26. figure;

27. sgtitle('Warisha Noushad - 400519903 & Jasmine Dosanjh - 400531879');

28. %For 100Hz Plot

29. subplot(2,2,1);

30. plot(nplot, xnT1(1:length(nplot)));

31. title('Frequency at 100Hz');

32. xlabel('Time (s)');

33. ylabel('Amplitude');

34. grid on;

35. %For 200Hz Plot

36. subplot(2,2,2);

37. plot(nplot, xnT2(1:length(nplot)));

38. title('Frequency at 200Hz');

39. xlabel('Time (s)');

40. ylabel('Amplitude');

41. grid on;

42. %For 400Hz Plot

43. subplot(2,2,3);

44. plot(nplot, xnT3(1:length(nplot)));

45. title('Frequency at 400Hz');

46. xlabel('Time (s)');

47. ylabel('Amplitude');

48. grid on;

49. %For 800Hz Plot

50. subplot(2,2,4);

51. plot(nplot, xnT4(1:length(nplot)));

52. title('Frequency at 800Hz');

53. xlabel('Time (s)');

54. ylabel('Amplitude');

55. grid on;

56. %Export the graph

57. exportgraphics(gcf, "Q2.jpg")

58. sound_created = [xnT1,xnT2,xnT3,xnT4];

59. % Save xnT as a wav sound file, soundfile.wav.

60. audiowrite('Lab3_Q2_soundwave.wav', sound_created, fs);

The output discrete-time sinusoids are shown below:

Figure 2. Discrete-Time Sinusoid at Varying Frequencies

The output sound file is 8 seconds long, with each 2-second segment playing a sinusoid of increasing input frequency in the order f=100, 200, 400, 800 Hz. As the input frequency increases, so does the output frequency, as the tone becomes higher-pitched and less deep/lower-pitched.

## Part 3.

The MATLAB code in Listing 3 again observes the effect on the discrete-time signal xn from Part 1 at varying frequencies. The code remains the same as Part 2, but is sampled at four new frequencies, f=7200, 7600, 7800, 7900 Hz.

Listing 3: Part 3

1. %% Question 3

2. % Sampling frequency and interval

3. fs = 8000;

4. Ts = 1/fs;

5. % Set time duration of plot, i.e., 10 msec.

6. tfinalplot = 10e-3;

7. % Make the time vector for the plot

8. nplot=0:Ts:tfinalplot;

9. % Make the time vector for replayed sound spurt

10. % Play the spurt for 2 seconds

11. tfinal = 2;

12. nsound=0:Ts:tfinal;

13. %Frequency for sin

14. f1 = 7200;

15. f2 = 7600;

16. f3 = 7800;

17. f4 = 7900;

18. % Sample the sinusoid.

19. xnT1 = sin(2*pi*f1*nsound);

20. xnT2 = sin(2*pi*f2*nsound);

21. xnT3 = sin(2*pi*f3*nsound);

22. xnT4 = sin(2*pi*f4*nsound);

23. %Plot Making

24. figure;

25. sgtitle('Warisha Noushad - 400519903 & Jasmine Dosanjh - 400531879');

26. %For 100Hz Plot

27. subplot(2,2,1);

28. plot(nplot, xnT1(1:length(nplot)));

29. title('Frequency at 7200Hz');

30. xlabel('Time (s)');

31. ylabel('Amplitude');

32. grid on;

33. %For 200Hz Plot

34. subplot(2,2,2);

35. plot(nplot, xnT2(1:length(nplot)));

36. title('Frequency at 7600Hz');

37. xlabel('Time (s)');

38. ylabel('Amplitude');

39. grid on;

40. %For 400Hz Plot

41. subplot(2,2,3);

42. plot(nplot, xnT3(1:length(nplot)));

43. title('Frequency at 7800Hz');

44. xlabel('Time (s)');

45. ylabel('Amplitude');

46. grid on;

47. %For 800Hz Plot

48. subplot(2,2,4);

49. plot(nplot, xnT4(1:length(nplot)));

50. title('Frequency at 7900Hz');

51. xlabel('Time (s)');

52. ylabel('Amplitude');

53. grid on;

54. %Export the graph

55. exportgraphics(gcf, "Q3.jpg")

56. sound_created = [xnT1,xnT2,xnT3,xnT4];

57. % Save xnT as a wav sound file, soundfile.wav.

58. audiowrite('Lab3_Q3_soundwave.wav', sound_created, fs);

The output discrete-time sinusoids are shown in Figure 3. These waveforms resemble the ones from Part 2 but appear in reverse order and with the opposite progression of frequency. Instead of having a higher frequency as the input frequency increases, the plotted signals output a lower frequency.

Figure 3. Discrete-Time Sinusoid at Higher Varying Frequencies

For the input frequencies f=7200, 7600, 7800, 7900 Hz. The output behaves unexpectedly. Instead of hearing higher-pitched tones when the input frequency increases, the output frequencies decrease, producing deeper, lower-pitched sounds. This is the opposite of what was observed in Part 2, where the input frequencies f=100, 200, 400, 800 Hz were all below the Nyquist limit, and therefore reproduced correctly (no alias).

This reversal occurs because all frequencies in Part 3 are above the Nyquist limit:

fNyquist=fs2=4000 Hz

Meaning they cannot be represented without aliasing. Since the input frequencies are close to the sampling frequency fs=8000 Hz, the sampled system wraps these high frequencies back into the valid range [0, 4000] Hz. These folding produces alias frequencies, calculated as:

7200-8000=800 Hz

7600-8000=400 Hz

7800-8000=200 Hz

7900-8000=100 Hz

Thus, although the input frequencies increase, their aliases move in the opposite direction, decreasing from 800 Hz down to 100 Hz. These are the same frequencies from Part 2, but in reverse. This is why the output sound becomes progressively deeper and lower in pitch.

## Part 4.

The performance of the telephone system would degrade significantly if anti-aliasing pre-filtering were not used, as sounds above 4000 Hz that enter the system would not be correctly replicated. As seen in Part 3, the aliasing would convert high-frequency input signals into a low-frequency output. This would cause distortions at the receiving end of the telephone, as the original input frequency is altered.

This filtering prevents these negative effects by removing all frequency components above 3.5 kHz, so that the signal contains no frequency above the Nyquist limit of 4 kHz. This ensures that significant aliasing does not occur, meaning that the sound at the receiving end will exhibit its expected behavior, as higher input frequencies will be transmitted at higher output frequencies.

If anti-aliasing filtering were applied in these experiments, the high-frequency signals from Part 3 (f=7200, 7600, 7800, 7900 Hz) would be filtered out entirely before sampling, since they lie above the filter cutoff of 3.5 kHz. As a result, none of these frequencies would reach the sampler, and the alias frequencies observed earlier would never appear. The output would therefore contain no audible noise (silence) for all four of the inputs.

# Question 2: Aliasing of a Frequency Chirp Signal

## Part 1.

The frequency-chirp signal used in this Question is defined as,

ct=cos⁡(πμt2+2πf1t+ϕ)

whose instantaneous frequency is,

ft=μt+f1

Listing 4: Part 1

In the script, the frequency is set to f1=100 Hz and the chirp rate μ=2000. A high sampling rate of fs=32000 Hz is chosen to ensure accurate representation of the chirp. The sampling period is          Ts=1fs=3.125×10-5 s. Two time vectors are created, t_short which produces the first 2000 samples of the chirp and t_long which generates the full 8-second chirp for audio playback. The chirp is then generated using chirp. Finally, the complete 8-second chirp is exported and plotted as a graph.

1. %% Question 5

2. % Frequency for sin

3. f5 = 100; % New frequency for Question 5

4. mu = 2000;

5. %Sampling frequency

6. fs = 32000; %Sampling rate

7. Ts = 1/fs; %Sampling Period

8. %Plot the first 2000 samples

9. nplot = 2000;

10. t_short = (0:nplot-1)*Ts;

11. %Plot for 8 second signal

12. t = 8;

13. t_long = 0:Ts:t;

14. %Create the chirp signal c(t)

15. chirp = cos(pi*mu*(t_long.^2) + 2*pi*f5*t_long);

16. % Plot the 2000 samples

17. figure;

18. plot(t_short, chirp(1:nplot));

19. title('Aliasing of Frequency Chirp Signal Q1');

20. text(0.0605,-0.5, 'Jasmine Dosanjh', 'FontSize',8)

21. text(0.0605,-0.6, '400531879', 'FontSize',8)

22. text(0.0605,-0.7, 'Warisha Noushad', 'FontSize',8)

23. text(0.0605,-0.8, '400519903', 'FontSize',8)

24. xlabel('Time (s)');

25. ylabel('Amplitude');

26. grid on;

27. %Export graph

28. exportgraphics(gcf, "Q5.jpg")

29. %Export audio

30. audiowrite('Lab3_Q5_soundwave.wav', chirp, fs);

The audio file began with a high frequency signal that increased until it could no longer be heard. The output signal is shown in Figure 4. It plots the first 2000 samples and its frequency increases with time (the period decreases). This graph follows the same trend as the audio clip.

Figure 4. Aliasing of Frequency-Chirp Signal f=100 Hz

## Part 2.

Listing 5: Question 2 for 16 kHz

The MATLAB code in Part 2 is identical to Part 1, but with the sampling frequency fs decreased from 32   kHz twice to observe the effects on the signal. Listing 5 is reduced by a factor of half where fs=16 kHz. Listing 6 is reduced by a half again where fs=8 kHz, which indicates what would happen if this signal were sent through the digital telephone network without anti-aliasing pre-filtering.

1. %% Question 5 for 16KHz

2. % Frequency for sin

3. f5 = 100; % New frequency for Question 5

4. mu = 2000;

5. %Sampling frequency

6. fs = 16000; %Sampling rate

7. Ts = 1/fs; %Sampling Period

8. %Plot the first 2000 samples

9. nplot = 2000;

10. t_short = (0:nplot-1)*Ts;

11. %Plot for 8 second signal

12. t = 8;

13. t_long = 0:Ts:t;

14. %Create the chirp signal c(t)

15. chirp = cos(pi*mu*(t_long.^2) + 2*pi*f5*t_long);

16. % Plot the 2000 samples

17. figure;

18. plot(t_short, chirp(1:nplot));

19. title('Aliasing of Frequency Chirp Signal at 16KHz');

20. text(0.122,-0.5, 'Jasmine Dosanjh', 'FontSize',8)

21. text(0.122,-0.6, '400531879', 'FontSize',8)

22. text(0.122,-0.7, 'Warisha Noushad', 'FontSize',8)

23. text(0.122,-0.8, '400519903', 'FontSize',8)

24. xlabel('Time (s)');

25. ylabel('Amplitude');

26. grid on;

27. %Export graph

28. exportgraphics(gcf, "Q5(16KHz).jpg");

29. %Export audio

30. audiowrite('Lab3_Q5(16kHz)_soundwave.wav', chirp, fs);

Listing 6: Question 2 for 8 kHz

1. %% Question 5 (8KHz)

2. % Frequency for sin

3. f5 = 100; % New frequency for Question 5

4. mu = 2000;

5. %Sampling frequency

6. fs = 8000; %Sampling rate

7. Ts = 1/fs; %Sampling Period

8. %Plot the first 2000 samples

9. nplot = 2000;

10. t_short = (0:nplot-1)*Ts;

11. %Plot for 8 second signal

12. t = 8;

13. t_long = 0:Ts:t;

14. %Create the chirp signal c(t)

15. chirp = cos(pi*mu*(t_long.^2) + 2*pi*f5*t_long);

16. % Plot the 2000 samples

17. figure;

18. plot(t_short, chirp(1:nplot));

19. title('Aliasing of Frequency Chirp Signal at 8KHz');

20. text(0.001,0.5, 'Jasmine Dosanjh', 'FontSize',8)

21. text(0.001,0.6, '400531879', 'FontSize',8)

22. text(0.001,0.7, 'Warisha Noushad', 'FontSize',8)

23. text(0.001,0.8, '400519903', 'FontSize',8)

24. xlabel('Time (s)');

25. ylabel('Amplitude');

26. grid on;

27. %Export graph

28. exportgraphics(gcf, "Q5(8KHz).jpg");

29. %Export audio

30. audiowrite('Lab3_Q5(8KHz)_soundwave.wav', chirp, fs);

The output signals are shown below:

Figure 5. Aliasing of Frequency-Chirp Signal f=8 kHz

Figure 6. Aliasing of Frequency-Chirp Signal f=16 kHz

For a frequency of 16 kHz, the first half of the audio file behaved identically to the 32 kHz audio as it began with a high frequency signal that increased but took half the original time to become inaudible. For the remaining half the sound reversed, and its frequency decreased back down again until it reached the beginning frequency. Whereas for a frequency of 8 kHz the audio file repeated the behaviour of the 16 kHz audio but at twice the rate for the first half of the audio file. Then it repeats for the remaining half.

In a telephone connection that includes anti-aliasing filtering, the high-frequency signals above the Nyquist limit would be filtered out entirely before sampling. This would produce a signal that begins with a high frequency signal and increases until it surpasses 3.5 kHz, becoming inaudible. As a result, there will be no component of the frequency that decreases as seen previously.

This final part consists of experimenting with other frequency values f1,  fs and chirp rates μ, and observing their effects on the audio signal. Recall that the instantaneous frequency of the chirp is,

ft=μt+f1

Increasing f1 causes the audio to begin at a higher frequency. As f1 becomes large enough that ft exceeds the Nyquist frequency, the signal aliases and the beginning of the audio jumps to a lower pitch. Conversely, decreasing f1 shifts the chirp downward, causing the signal to begin at a lower frequency. This behaviour follows directly from the linear dependence of instantaneous frequency on f1.

As the sampling rate fs decreases, the output behavior is identical to Part 1. It begins with a high-frequency signal that increases and decreases, with this pattern repeating multiple times. This behaviour is due to aliasing, as the output signal distorts as fs decreases below the Nyquist sampling rate of the original signal. As fs As the increases, the output sounds more like a true chirp: smooth, continuous, with fewer repeats and a gradual decrease throughout. This behavior is due to the sampling frequency being high enough to reproduce the input signal, causing less aliasing.

As the chirp rate μ decreases, the sound alternates from high to low frequencies at a slower rate. When μ increases, the frequency signal has a higher rate of increase and decrease, causing the signal to repeat itself more. This is due to μ being the slope (rate of change) of the instantaneous frequency of the signal.